# LKA Analysis Report

## 1. Overview

The Lane Keep Assist (LKA) system was evaluated on two different track configurations consisting of:

- Straight road sections

- Variable-curvature curved sections

The controller autonomously maintained lateral steering while the simulation driver handled longitudinal control.

The algorithm used:

- Proportional-Derivative (PD) lateral control

- Lane-centre calibration using tRoad

- Steering saturation limits

- Deadband-based steering suppression

- Filtered derivative response for stability

# 2. Track Comparison Analysis

## Track Configuration 1

### Observations

- Initial transient deviation occurred during the transition into the curved section.

- Maximum lateral deviation reached approximately:

0.2747 m

- Worst deviation occurred near:

11.20 s

- After the transient response, the vehicle stabilised and maintained lane position.

### Behaviour Analysis

The controller initially over-corrected when entering the curve due to:

- sudden curvature change

- steering response delay

- derivative oscillation

However:

- oscillations reduced gradually

- the controller converged back toward lane centre

- the vehicle remained within legal lane boundaries

### Performance

| Metric | Result |
| --- | --- |
| RMSE | 0.0604 m |
| ILC | 100 % |
| Lane Exit | No |
| Status | PASS |

# Track Configuration 2

### Observations

- Larger transient oscillations occurred during curvature transitions.

- Maximum deviation increased to:

0.3220 m

- Worst deviation occurred at:

10.88 s

- Oscillation frequency increased in the curved region.

### Behaviour Analysis

Compared to Track 1:

- sharper curvature caused higher steering demand

- controller produced stronger corrective oscillations

- derivative term amplified curvature-entry response

Despite the higher oscillation:

- vehicle recovered successfully

- lane boundaries were never exceeded

- no DNF condition occurred

### Performance

| Metric | Result |
| --- | --- |
| RMSE | 0.0634 m |
| ILC | 100 % |
| Lane Exit | No |
| Status | PASS |

# 3. Greatest Deviation Zones

The greatest lateral deviations occurred:

- during straight-to-curve transitions

- immediately after entering variable-curvature sections

Reason:

- sudden change in road heading

- delayed steering convergence

- derivative overshoot

The controller required a short stabilisation interval before converging back toward lane centre.

# 4. Algorithm Response to Curvature Changes

The algorithm responded using:

- proportional correction based on lateral error

- derivative damping based on rate of lateral deviation change

### Controller Characteristics

| Behaviour | Observation |
| --- | --- |
| Straight sections | Stable lane-centering |
| Mild curves | Small oscillations |
| Sharp curves | Temporary overshoot |
| Recovery | Stable convergence |
| Steering saturation | Prevented aggressive instability |

The derivative filtering (ALPHA) helped reduce high-frequency oscillations during curvature transitions.

# 5. Compliance with Competition Constraints

## RMSE Requirement

Competition threshold:

Penalty begins beyond ±0.1 m

Observed:

- RMSE remained below threshold in both runs

- controller maintained acceptable lateral accuracy

PASS.

## ILC Requirement

Requirement:

Vehicle must remain within lane boundaries

Observed:

ILC = 100 %

The vehicle remained within legal lane boundaries throughout both runs.

PASS.

## DNF Condition

Requirement:

Front axle exits lane and fails to return within 5 seconds

Observed:

- no lane exit occurred

- vehicle recovered after all transient deviations

PASS.