# Stage-by-Stage Performance Analysis — Endurance Event

## Team Details

- Team Name: The Elecruisers

- Team ID: 264022

# 1. Stage 1 — Curved Road Navigation

## Objective

The ego vehicle was required to safely navigate variable-curvature road sections while maintaining lane position.

## System Behaviour

During curved sections:

- the lane error increased temporarily due to road curvature,

- the derivative steering component compensated for rapid changes,

- and filtered steering reduced oscillatory corrections.

The controller adapted steering continuously according to the changing road curvature.

## Performance Observations

| Parameter | Observation |
| --- | --- |
| Curve Tracking | Stable |
| Steering Smoothness | Controlled |
| Overshoot | Minor |
| Lane Departure | None |
| Vehicle Stability | Maintained |

## Result

PASS

The ego vehicle successfully navigated curved road sections while maintaining lane tracking.

# 2. Stage 2 — Straight Lane Keeping

## Objective

The ego vehicle was required to autonomously maintain its position at the center of the lane while travelling on a straight road section.

## System Behaviour

The Lane Keeping Assist (LKA) controller continuously monitored the lateral deviation (Vhcl.tRoad) from the lane center and applied steering corrections using a PD-based steering controller.

The steering controller used:

- proportional correction for lane offset,

- derivative correction for stability,

- and low-pass filtering for noise reduction.

The controller equation used was:

Steering = Kp × LaneError + Kd × ErrorRate

The ego vehicle maintained stable lane tracking with minimal oscillations during straight road navigation.

## Performance Observations

| Parameter | Observation |
| --- | --- |
| Lane Center Tracking | Stable |
| Steering Oscillations | Minimal |
| Vehicle Stability | Maintained |
| Average Lane Error | Low |
| Speed Stability | Maintained near 30 km/h |

## Result

PASS

The vehicle successfully maintained lane discipline during the straight section.

# 3. Stage 3 — Dynamic Object Insertion (AEB)

## Objective

The ego vehicle was required to:

- detect a cut-in vehicle,

- activate emergency braking,

- stop safely,

- and resume stable operation after threat handling.

## System Behaviour

The AEB controller used radar sensor inputs to continuously monitor:

- target distance,

- relative velocity,

- lateral displacement,

- and TTC (Time-To-Collision).

The cut-in vehicle was detected when:

|DistY| < Lane Threshold

Once the target entered the ego lane:

- TTC was calculated,

- braking logic became active,

- throttle was disabled,

- and progressive braking was applied.

## Braking Behaviour

The braking profile was distance-dependent:

- mild braking at higher distances,

- moderate braking at intermediate TTC values,

- stronger braking near stopping distance.

This prevented:

- collisions,

- excessive jerk,

- and unstable stopping.

## Performance Observations

| Parameter | Observation |
| --- | --- |
| Cut-In Detection | Successful |
| AEB Trigger | Immediate |
| Collision Avoidance | Successful |
| Deceleration Behaviour | Controlled |
| Final Stop | Stable |
| TTC Monitoring | Functional |

## Result

PASS

The ego vehicle successfully detected the cut-in target and safely executed emergency braking.

# 4. LKA ↔ AEB Mode Transition Analysis

## Transition Requirement

The endurance event required smooth integration between:

- lateral lane control (LKA),

- and longitudinal emergency braking (AEB).

The system had to:

- maintain lane stability,

- while simultaneously handling obstacle avoidance braking.

## Transition Behaviour

### Before AEB Activation

- LKA maintained lane-centre tracking.

- Speed controller maintained ~30 km/h cruising speed.

### During AEB Activation

- AEB overrode throttle commands.

- Brake commands became dominant.

- LKA steering remained active to preserve lane stability.

### After Threat Handling

- Vehicle stabilized safely without lane departure.

- No steering instability occurred during braking.

## Integrated Controller Performance

| Feature | Result |
| --- | --- |
| LKA Stability During Braking | Stable |
| Steering Oscillation During AEB | Minimal |
| Simultaneous Lateral + Longitudinal Control | Successful |
| Control Conflict | None |
| Vehicle Stability | Maintained |