# Difficulties Faced During Development

# CarMaker API Compatibility Issues

## Problem

Multiple runtime errors occurred due to differences between:

- expected CarMaker Python API methods,

- and the installed CarMaker version.

### Errors Faced

AttributeError: 'CarMaker' object has no attribute 'get'

AttributeError: 'Quantity' object has no attribute 'value'

TypeError: float() argument must be a string or a real number, not 'Quantity'

## Cause

Different CarMaker API versions use different data access methods.

Initially attempted methods included:

- .value

- .Value

- .get()

- float(quantity)

However, the installed version required:

cm.read()

quantity.data

## Solution

The signal-reading logic was corrected using:

cm.read()

value = quantity.data

This resolved all API compatibility errors.

# 5.2 LKA Steering Oscillations

## Problem

The ego vehicle initially showed:

- unstable steering corrections,

- oscillatory lane tracking,

- and excessive lateral movement.

## Cause

High proportional steering gain caused aggressive steering behaviour.

## Solution

The steering controller was tuned using:

- lower proportional gain,

- derivative damping,

- low-pass filtering,

- and steering limits.

This stabilized lane tracking behaviour.

# 5.3 AEB False Triggering

## Problem

The AEB system occasionally activated even when:

- the target vehicle was still outside the ego lane.

## Cause

Improper lateral threshold tuning for cut-in detection.

## Solution

Lane-based target filtering was added:

target_in_lane = abs(ydist) < LANE_THRESHOLD

This ensured braking occurred only for valid in-lane threats.

# 5.4 Excessive Deceleration Spikes

## Problem

The deceleration graph initially contained:

- unrealistic spikes,

- unstable braking peaks,

- and noisy values.

## Cause

Raw differentiation of speed values amplified noise.

## Solution

Median filtering and clipping were added:

decel_smooth = medfilt(decel_log, kernel_size=5)

This produced smoother deceleration plots.

# 5.5 Incorrect Stopping Distance

## Problem

The ego vehicle sometimes:

- stopped too early,

- or overshot the target vehicle.

## Cause

Brake thresholds were not synchronized with:

- target distance,

- TTC,

- and closing speed.

## Solution

Progressive braking stages were tuned iteratively based on:

- TTC,

- relative velocity,

- and stopping distance.

This improved stopping accuracy.

# 5.6 GUI Plot Integration Difficulties

## Problem

Initially:

- plots were generated separately,

- layout was cluttered,

- and BEV trajectory integration failed.

## Solution

A combined GUI layout was implemented containing:

- Speed vs Time

- Lateral Deviation

- TTC

- Distance

- Brake

- Deceleration

- BEV trajectory

This improved readability and presentation quality.

# 5.7 Combined LKA + AEB Integration Challenges

## Problem

Integrating:

- lane control,

- speed control,

- and emergency braking

created conflicts between controllers.

Examples:

- steering instability during braking,

- throttle not disengaging during AEB,

- simultaneous gas and brake commands.

## Solution

Priority-based control handling was implemented:

- AEB overrides throttle during emergency braking,

- LKA remains active for lane stability,

- brake commands take highest priority.

This ensured stable combined controller behaviour.

# Final Outcome

The final Endurance implementation successfully:

- maintained stable lane tracking,

- navigated curved roads,

- detected cut-in threats,

- activated emergency braking,

- maintained stability during braking,

- and generated all required outputs and analysis plots.

# Final Status

PASS