# Difficulties Faced During LKA Development

## 6.1 Incorrect CarMaker Signal Variables

### Problem

Several variables initially used for lane position measurement were invalid or unsupported:

- Car.Road.tRoad

- Car.Road.t

This caused runtime errors:

no such element in array

### Solution

The correct signal:

Vhcl.tRoad

was identified and used successfully.

# 6.2 Steering Direction Inversion

### Problem

Initial steering commands caused the vehicle to:

- steer opposite to required correction

- leave the road rapidly

### Cause

Incorrect steering sign convention.

### Solution

A steering direction multiplier:

STEER_SIGN = +1 / -1

was introduced and tuned experimentally.

# 6.3 Excessive Oscillation

### Problem

The vehicle oscillated heavily:

- crossing multiple lanes

- over-correcting left/right

- unstable during curves

### Cause

High proportional gain and noisy derivative response.

### Solution

The following improvements were implemented:

- reduced proportional gain

- reduced derivative gain

- derivative low-pass filtering

- steering saturation limits

- deadband region near lane centre

# 6.4 Immediate Off-Road Behaviour

### Problem

Vehicle immediately exited the lane after autonomous control activation.

### Cause

Controller began steering before proper lane-centre calibration.

### Solution

A calibration phase was added:

- multiple tRoad samples collected

- median lane-centre value computed

- lateral control enabled only after calibration completed

# 6.5 HTML Graph Output Issue

### Problem

Matplotlib generated HTML/browser outputs instead of popup graph windows.

### Cause

Backend mismatch in WSL/Linux environment.

### Solution

Backend was forced manually:

matplotlib.use("TkAgg")

This enabled real-time popup graph windows similar to the AEB implementation.

# 6.6 Curvature Transition Instability

### Problem

Vehicle showed strongest instability during:

- straight-to-curve transitions

- variable curvature sections

### Cause

Rapid heading error changes caused temporary steering overshoot.

### Solution

Controller tuning was performed:

- filtered derivative term

- reduced steering limits

- smaller deadband

- smoother damping behaviour

# 7. Final Conclusion

The developed LKA system successfully:

- maintained autonomous lane centering

- handled straight and curved road sections

- remained within lane boundaries

- achieved 100% in-lane coverage

- avoided DNF conditions

- maintained RMSE below penalty thresholds

The controller demonstrated stable recovery behaviour after transient curvature disturbances and complied with the required competition performance criteria.