## Written Analysis — Automatic Emergency Braking (AEB)

### Team Details

- Team Name: The Elecruisers

- Team ID: 264022

# 1. Detection Method

The AEB system detects a collision threat using a forward radar sensor mounted on the ego vehicle. The algorithm continuously monitors:

- Relative distance to the target vehicle

- Relative velocity

- Lateral offset of the detected object

- Time-To-Collision (TTC)

The target vehicle initially travels in an adjacent lane and then performs a cut-in manoeuvre into the ego vehicle’s lane. The system identifies the cut-in when the lateral distance of the detected object falls within the ego lane threshold.

The detection logic used is:

target_in_lane = abs(ydist) < LANE_THRESHOLD

Once the target enters the ego lane and the longitudinal distance reduces below the configured threshold, the AEB controller becomes active.

The braking decision is based on:

- Distance to obstacle

- TTC value

- Relative closing speed

The braking intensity is progressively increased as the ego vehicle approaches the target vehicle.

# 2. Sensors Used

The following virtual radar sensor signals from IPG CarMaker were used:

| Sensor Signal | Purpose |
| --- | --- |
| Sensor.Radar.Vhcl.RAD00.Obj0.Dist | Measures longitudinal distance to target vehicle |
| Sensor.Radar.Vhcl.RAD00.Obj0.Vrel | Measures relative velocity between ego and target |
| Sensor.Radar.Vhcl.RAD00.Obj0.DistY | Measures lateral displacement of target |
| Car.v | Ego vehicle speed |
| Car.tx, Car.ty | Ego vehicle BEV trajectory position |

### Sensor Fusion Logic

The controller combines:

- longitudinal distance,

- relative velocity,

- and lateral lane position

to determine whether the object is:

- in the ego lane,

- approaching dangerously,

- and requires emergency braking.

# 3. Response Latency

The response latency is defined as the time difference between:

- Target entering the ego lane and satisfying the detection condition

- Initial brake command activation

From the simulation results:

| Event | Approx Time |
| --- | --- |
| Cut-in detection / AEB trigger | ~16.1 s |
| Initial brake application | ~16.1–16.2 s |

### Estimated Response Latency

≈ 0.1 s

The low latency was achieved because the controller continuously polls radar data in real time inside the control loop.

# 4. Zone-wise Behaviour Analysis

## Zone 1 — Acceleration Zone

### Requirement

30 km/h ±3 km/h

### Observed

≈ 31.1 km/h

### Result

PASS

The ego vehicle successfully accelerated from rest and stabilized near the target speed before the cut-in occurred.

## Zone 2 — Threat Detection Zone

### Requirement

Deceleration between 5–8 m/s²

### Observed Peak Deceleration

≈ 7.2 m/s²

### Result

PASS

The radar sensor successfully detected the target vehicle entering the ego lane. The controller activated braking while maintaining the required deceleration constraint.

## Zone 3 — Stopping Zone

### Requirement

Final stopping distance = 3–9 m

### Observed

≈ 5.95 m

### Result

PASS

The ego vehicle stopped safely without collision while maintaining the required stopping distance window.

# 5. BEV Trajectory Analysis

The Bird’s Eye View trajectory plot shows:

- Ego vehicle longitudinal motion

- Constant lane tracking

- AEB trigger point

- Final stopping point

- Velocity colour mapping

The velocity-coloured trajectory demonstrates:

- constant cruising during Zone 1,

- controlled deceleration during Zone 2,

- and complete stop in Zone 3.

# 6. Brake Force / Deceleration Behaviour

The brake controller applies progressive braking:

- low brake at higher distances,

- medium brake as TTC decreases,

- stronger braking near stopping condition.

This prevents:

- sudden instability,

- excessive jerk,

- and collision risk.

The final deceleration profile remains compliant with the required:

5–8 m/s²

# 7. Final Scenario Outcome

| Criterion | Result |
| --- | --- |
| Ego initiates movement | PASS |
| Reaches 30 km/h ±3 km/h | PASS |
| Detects cut-in vehicle | PASS |
| Deceleration within 5–8 m/s² | PASS |
| Stops within 3–9 m | PASS |
| Collision avoided | PASS |

# Final Status

PASS