Sensor Configuration File

LKA System — aBAJA SAEINDIA 2026  |  Software Virtual World Simulation

Team Elecruisers, SRM Institute of Science and Technology

Vehicle: .tmp_vehicle_startpos_Rajath_Rayzan

# 1. Overview

This document provides the complete sensor configuration used in the LKA simulation for the aBaja SAEINDIA 2026 competition. It covers all sensors mounted on the vehicle, their parametric settings as configured in CarMaker Office, physical mounting positions, and relevant DVA (Data Variable Access) quantities. All sensor inputs are raw or minimally processed data — no ground-truth lane state is used as an algorithm input (compliant with Rule R1).

The vehicle data set is configured in CarMaker Office under the file .tmp_vehicle_startpos_Rajath_Rayzan. Four sensor types are active: Inertial (IN00), Road (RD00), Radar (RAD00), and Camera RSI (CAM00).

# 2. Sensors — Summary

| Sensor Name | Type | Parametrization | Mount Position x/y/z [m] | Cycle Time [ms] |
| --- | --- | --- | --- | --- |
| IN00 | Inertial | Inertial - IN00 | 1.2 / 0.0 / 0.6 | — |
| RD00 | Road | Road - RD00 | 1.5 / 0.0 / 0.3 | — |
| RAD00 | Radar | Radar - RAD00 | 1.8 / 0.0 / 0.5 | 50 |
| CAM00 / Camera_RGB | Camera RSI | CameraRSI - CAM00 | 1.6 / 0.0 / 1.5 | 33 |

Note: All sensors mounted to Frame Fr1A. Orientation x/y/z = 0.0/0.0/0.0 deg except Camera_RGB (z = -0.2 deg). Body mounting: Frame Fr1A for all.

# 3. Inertial Sensor — IN00

The Inertial sensor (IN00) provides vehicle body dynamics data including velocity, acceleration, and yaw rate. It is mounted at position x=1.2 m, y=0.0 m, z=0.6 m on Frame Fr1A with zero orientation. Calculation class is set to Global.

| Parameter | Value | Notes |
| --- | --- | --- |
| Sensor Name | IN00 | Inertial Sensor |
| Mount Position x/y/z [m] | 1.2 / 0.0 / 0.6 | Frame Fr1A |
| Orientation x/y/z [deg] | 0.0 / 0.0 / 0.0 | No rotation |
| Calculation Class | Global | Standard global reference frame |
| Active | Yes | Enabled in Assembly |

# 4. Road Sensor — RD00

The Road sensor (RD00) provides road geometry and deviation data along the route reference line. It is used for post-run validation only — DVA road deviation quantities are NOT read by the control algorithm (Rule R1 compliant).

| Parameter | Value | Notes |
| --- | --- | --- |
| Sensor Name | RD00 | Road Sensor |
| Mount Position x/y/z [m] | 1.5 / 0.0 / 0.3 | Frame Fr1A |
| Orientation x/y/z [deg] | 0.0 / 0.0 / 0.0 | No rotation |
| Preview Distance [m] | 10 | Look-ahead distance |
| Preview Mode | Along Route Reference Line | Overrides sensor position |
| Consider Bumps | No | Unchecked |
| Algorithm Use | Post-run validation only | NOT read by control pipeline |

# 5. Radar Sensor — RAD00

## 5.1 Basic Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Sensor Name | RAD00 | Radar Sensor |
| Mount Position x/y/z [m] | 1.8 / 0.0 / 0.5 | Front-center, Frame Fr1A |
| Orientation x/y/z [deg] | 0.0 / 0.0 / 0.0 | Forward-facing |
| Cycle Time [ms] | 50 | 20 Hz update rate |
| Latency min/max [ms] | 1.0 / 1.0 |  |
| Field of View h [deg] | 120 | Horizontal FoV |
| Field of View v [deg] | 30.0 | Vertical FoV |
| Range min / max [m] | 1 / 160 | Detection range |
| Objects max [-] | 32 | Max trackable objects |

## 5.2 Additional (RF) Parameters

| Parameter | Accuracy | Resolution | Notes |
| --- | --- | --- | --- |
| Distance [m] | 0.3 | 0.7 |  |
| Azimuth [deg] | 3 | 5 |  |
| Speed [km/h] | 0.1 | 0.2 |  |
| Separability [-] | — | 0.7 |  |

| RF Parameter | Value | Notes |
| --- | --- | --- |
| Transmit Frequency [GHz] | 76 | Automotive 77 GHz band |
| Transmit Power [dBm] | 37 |  |
| System Losses [dB] | 0.0 |  |
| Noise Bandwidth [Hz] | 50000 | 50 kHz |
| Noise Figure [dB] | 10 |  |
| Probability of Detection min | 0.5 |  |
| Probability of False Alarm | 10e-6 |  |
| Antenna Gain Map | Radar_Default |  |
| False Positives | Disabled | Unchecked |

# 6. Camera RSI Sensor — CAM00

## 6.1 Sensor Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Sensor Name | CAM00 (Camera_RGB) | Camera RSI Sensor |
| Mount Position x/y/z [m] | 1.6 / 0.0 / 1.5 | Frame Fr1A |
| Orientation x/y/z [deg] | 0.0 / 0.0 / -0.2 | Slight downward tilt (z) |
| Resolution h/v [px] | 640 × 480 | VGA resolution |
| FoV h [deg] | 75 | Horizontal field of view |
| Clipping Plane near/far [m] | 0.1 / 4000.0 |  |
| Coloration Mode | Realistic |  |
| VIB Mode | Disabled | Unchecked |
| Principal Point Offset x/y [m] | 0.0 / 0.0 | No offset |
| Cycle Time [ms] | 33 | ~30 Hz |
| Latency min/max [ms] | 1.0 / 1.0 |  |

## 6.2 Sensor Cluster Configuration

The Camera RSI sensor is managed through the Sensor Cluster Configuration (automatically clustered). The cluster streams raw RGB frames over TCP for external processing.

| Cluster Parameter | Value | Notes |
| --- | --- | --- |
| Cluster Name | Cluster CameraRSI | Automatically clustered |
| Sensor in Cluster | Camera_RGB |  |
| Cycle Time [ms] | 33 | ~30 Hz |
| Cycle Offset | 0 |  |
| Latency min/max [ms] | 1.0 / 1.0 |  |
| Use Movie NX | Enabled | Checked |
| Client Type | User Defined | Raw signal data stream |
| Socket Port | 2210 | TCP localhost:2210 |
| Client Connect Lead Time [s] | 0 |  |
| GPU Assignment | GPU 0 (Default) — rajath | tag: Default |

# 7. DVA (Data Variable Access) Quantities

The following CarMaker DVA quantities are read by the algorithm at each control tick for vehicle state estimation. These are standard simulator state outputs and do NOT include any lane-position or road-deviation ground truth (Rule R1 compliant).

| DVA Quantity | Unit | Use in Algorithm | R1 Status |
| --- | --- | --- | --- |
| Vhcl.v | m/s | Vehicle speed — used to scale Pure Pursuit lookahead and PID output | Compliant |
| Car.YawRate | rad/s | Yaw rate — logged to CSV for post-run analysis | Compliant |
| Car.ay | m/s² | Lateral acceleration — logged to CSV for analysis | Compliant |
| Road Sensor DVA (RD00) | — | Available in model but NOT read by algorithm — post-run validation only | Compliant |

# 8. Compliance with Submission Rules

| Rule | Requirement | Implementation | Status |
| --- | --- | --- | --- |
| R1 | No ground-truth object states as algorithm inputs | Camera RSI raw frames only; DVA lane quantities not read | ✔ COMPLIANT |
| R2 | No simulator-internal APIs exposing privileged world state | APO interface used for control outputs only; no scene graph access | ✔ COMPLIANT |
| R3 | Algorithm inputs from configured sensor models only | All perception from Camera RSI TCP stream | ✔ COMPLIANT |
| R4 | Approved sensor types only (Camera, LiDAR, RADAR, IMU, GNSS) | Camera RSI + Radar + Inertial — all approved types | ✔ COMPLIANT |
| R5 | Sensor config validated by Technical Review before presentation | Submit this document to Technical Committee | ■ PENDING |
| B.1 | Real-world reproducibility — no simulation-only shortcuts | BEV + Sliding Window pipeline deployable on embedded hardware | ✔ COMPLIANT |
| B.5 | Min real-time rates: Perception ≥ 10 Hz, Control ≥ 20 Hz | Full pipeline runs at 30 Hz in CarMaker | ✔ COMPLIANT |

Note: Rule R5 status is PENDING until Technical Committee review is complete. Submit this document along with simulation run logs.